7/31/2014 – By Harlyn Oman

This script file changes the operation of the GHM bucket system so that the hopper door is closed by default.  It is to be used when customers want to harvest with 2-plots on the machine at a time.  Both script files must be copied to: “C:\HarvestMaster\Mirus\Config\Generic\Scripts”

The “RB-“ on the front-end of the file must be removed to activate them.  You can have the customer rename the originals before removing the “RB-.”  This will allow you to easily go back to the original file if needed.

To harvest with these edited script files, the customer MUST remember to:

- Harvest through first plot and let accumulate in cyclone.

- Cycle the hopper door manually (dumping plot in weigh bucket) or press the cycle button and let the hopper cycle.

- If you chose to press the cycle button, you MUST relocate to the plot just harvested and overwrite data in that plot (should be zero).

NOTE: If taking notes, the operator MUST collect the 2nd plot notes before relocating.